Links to purchase Sensors:

- GM Genuine Parts 13507128 Flex Fuel Sensor - https://www.amazon.in/ACDelco-13507128-Original-Equipment-Sensor/dp/B073ZQ1G2P/ref=pd_bxgy_thbs_d_sccl_1/257-4142002-6851454?psc=1

- PT-E85-2 Flex Fuel Sensor Connector Pigtail, 1ft (12"/30.5cm) Cable, GM E85 Compatible, Ethanol Sensor Connector, Automotive, Ethanol Level Monitoring - https://www.amazon.in/Xialukeparts-Connector-Compatible-Automotive-Monitoring/dp/B0BFW67BC7

- BMI323 : 7Semi BMI323 IMU Breakout Board 6DOF Accelerometer Gyroscope Qwiic SPI – Robocraze

- WiF: NewYall WiF Water in Fuel Sensor with Drain : Amazon.in: Home Improvement

- DS18B20 (tutorial on 1-wire: https://www.youtube.com/watch?v=fBkfa_L9C5g )

- Turbidity Sensor Module for Suspended Solids Detection https://robu.in/product/turbidity-sensor-suspended-turbidity-value-detection-module-kit/

Parameters we are monitoring:

- Ethanol %  Pulse width (DC)

- Water in Fuel  Analog

- Solid suspension using turbidity  Analog

- IMU – BMI323 6-axis IMU with a 3-axis accelerometer and 3-axis gyroscope, supporting SPI, I²C, and I3C. It can sample up to 6.4 kHz and is well suited for driver behavior analysis

- Fuel Density –  interface unknown

- Fuel Temperature DS18B20  1-wire

Other components required:

- Protoboard to connect the sensors

- Male type Pin headers

- Enclosure IP68/66 graded

- Jumper wires

- Battery for powering up

- Charging Management

- Bluetooth App to display the data

- Petrol for calibration & testing

Misc. Links:

- Uno-Q Documents: https://docs.arduino.cc/hardware/uno-q/

- Example AI projects already done: https://projecthub.arduino.cc/?value=UNO+Q

Requirements:

- The MCU STM32U585 will fetch the data from the sensors

| Sensor | Interface | Acquire |
| --- | --- | --- |
| 13507128 Flex Fuel Sensor | Pulse width | Ethanol% |
| IMU | I2C | 3AA & 3AG |
| WiF | Analog | Water % |
| Turbidity | Analog | Solid particulate |
| DS18B20 | 1-wire | Fuel Temp |
| Fuel Densit | <to be decided> | Fuel density |

The MCU will run either with some RTOS or bare-metal code in C to acquire these data.

- Data in the MCU will run @ 100 mS sampling, the stored data will be in a local buffer. Each sensor shall have its own buffer space to maintain data integrity.

- Cleaned data will be stored back into the same buffer.

- This data will be transferred to the MPU using proper IPC technique.

- MCU will send data in packets in form below:
|SOF=5A|LENGTH OF DATA|DATA BUFFER|CRC16|EOF=7F|
These packets will flow from the MCU to the MPU over the connection bridge UART1:
Details here:

On the Arduino UNO Q, there are two processors:

MPU: Qualcomm Dragonwing QRB2210 running Debian Linux

MCU: STM32U585 running Zephyr/Arduino sketches

From Arduino's documentation, the MPU and MCU communicate through the Arduino Router / Bridge RPC framework. The Linux side talks to an arduino-router daemon using MessagePack-RPC, and the MCU exposes functions through the Bridge library

The official documentation  emphasizes the RPC layer rather than the hardware bus. However, Arduino examples and community investigations show that:

- The standard Bridge communication uses an internal UART link.

- On Linux this appears as /dev/ttyHS1.

- On the STM32 side it is Serial1.

- Arduino reserves this channel for the Router/Bridge and recommends not using it directly.

TO MPU

Arduino-bridge RPC

RPC Data path to MPU

STM32 Global Buff Mgr. (gBuff[NO_OF_SNSRS])

Application Data Filtering Apps

Cb invokes sensor call

Application Middleware – Reads sensors

Intrpt Vec.

init

HAL-GPIO

Timer GPT

HAL-ADC

HAL-SPI

HAL-UART

HAL-I2C

RTOS – FreeRTOS/ Zyphr / Bare Metal Code

STM32U585

Software Architecture:

Responsibility:

- Turjasu – To handle all MCU side comms, data processing and sending to MPU

- Siddhartha – To handle the RPC, MPU-MCU interconnect, MPU side bootup, scheduling, IPC, Data connectivity to mobile, end-to-end integrity of the project

- Sulagna – AI model running on the MPU, how we can utilise the resources on the MPU side, BT app, integration with the MPU, GPU and ensure complete function of the AI model alongwith endurance, repeatability, sustainability, accuracy (acceptable), storage of data in the flash/eMMC

For any help Sulagna should reach to Siddhartha, if required Turjasu will pitch in.

WE HAVE TO DO IT!! IT IS A VERY GOOD PROJECT FOR ALL OF US

Hardware Pin Connections:

DS18B20

RTC

WiF (analog)

IMU

PA4 – A0						   PB10,11 I2C2

Turbidity (analog)

PA5 – A1

UNO-Q

MCU

Optional SPI Flash

UNO-Q

GPU

UNO-Q

MPU

PB2/4 1wire

PB13,14,15,9 SCK,MISO,MOSI,SS’

References:

- Working with Arduino Router RPC : https://docs.arduino.cc/tutorials/uno-q/routerbridge-multilanguage/